# CHAPTER 01 - Influence Lines of Determinate and Indeterminate Structures

## I. Influence Lines of Determinate Structures

### I.1. Introduction

### I.2. Influence lines for beams: Static–analytical approach

### I.3. Müller–Breslau principle: kinematic–qualitative approach

### I.4. Influence lines for Girders with floor systems

### I.5. Influence lines for Trusses

### I.6. Influence lines for Frames

### I.7. Applications of influence lines

#### I.7.1. Response at a particular location due to a single moving concentrated load

#### I.7.2. Response at a particular location due to a uniformly distributed live load

#### I.7.3. Response at a particular location due to a series of moving concentrated loads

#### I.7.4. Absolute maximum response

### I.8. Problems

## II. Influence Lines of Indeterminate Structures

Influence Lines of determinate and Indeterminate Structures

- Introductoion

Influence lines have importante application for the design of structures that resist large live loads. In this module we will discuss how to draw the influence line for a statically determinate structure. The theory is applied to structures subjected to a distributed load or a series of concentrated forces, and specific applications to floor girders and bridge trusses are given. The determination of the absolute maximum live shear and moment in a member is discussed at the end of this module

- Learning outcomes: apply the methods of structural analysis for external and internal actions on various types of structural systems due to static and moving loads.

- Learning objectives

Student learning objectives for this module upon completion of this module, you will be able to:

Know the meaning of influence lines.

Determine its an important part in the design of bridges, industrial crane rails, conveyors, and other structures where loads move across their span.

- Learning activities

Activities for this module

Do the assignments and problem sets

Take the quiz (Quiz1)

- Influence Lines of determinate Structures

I.1. Defintion

An influence line represents the variation of the reaction, shear, moment, or deflection at a specific point in a member as a concentrated force moves over the member. Once this line is constructed, one can tell at a glance where the moving load should be placed on the structure so that it creates the greatest influence at the specified point. Furthermore, the magnitude of the associated reaction, shear, moment, or deflection at the point can then be calculated from the ordinates of the influence line diagram. For these reasons, influence lines play an important part in the design of bridges, industrial crane rails, conveyors, and other structures where loads move across their span.

The following sign conventions are used:

Table 1. sign conventions

- Remark: For other sign conventions, they must be clearly defined.

I.2 Influence Lines for Beams

I.2.1Procedure for Analysis(Analytical Method)

Either of the following two procedures can be used to construct the influence line at a specific point P in a member for any function (reaction, shear, or moment). For both of these procedures we will choose the moving force to have a dimensionless magnitude of unity.

I.2.1 Tabulate Values

- Place a unit load P=1at different positions xalong the member.

- For each position, use statics to calculate the required quantity: reaction R, shear force V, or bending moment M.

- Apply the adopted sign conventions.

- For a statically determinate beam, the influence line consists of straight-line segments.

- Prepare a table showing the unit-load position xand the corresponding value of R, V, or M.

- Plot the calculated values and connect the points to construct the influence line.

Key principle:

Example 01

Construct the influence line for the vertical reaction at Aof the beam shown in Fig. 1 using the analytical method by tabulating the values.

Fig 1

SOLUTION

Tabulate Values. A unit load is placed on the beam at each selected point x and the value of Ay is calculated by summing moments about B. For example, when x=2.5 m, x=5 m and x=7 m, see Figs. 2-3 and 4, respectively. The results for Ay are entered in the Table 2. A plot of these values yields the influence line for the reaction at A, Fig. 5. This line or diagram gives the reaction at A as the unit load moves from one position to the next along the beam.

Table 2

Fig 4

Fig 3

Fig 2

Fig 4

Fig 5

I.2.2 Influence-Line Equations

Rather than placing the unit load at specific points on the beam and plotting the values of Ay for each point, we can place the unit load a variable distance x from A, Fig 6. The reaction Ay as a function of x can be determined from.

Influence Line Equation

The equation expressing the reaction Ay as a function of the position x of the unit load (P=1) on the 10 m beam is:

Ayx=1-110x for 0≤x≤10 m

Step-by-Step Derivation Method

- Definition of distances :

  - The total span of the beam is L=10 m.

  - The unit load P=1 is located at a distance x from support A.

  - The lever arm of the load P relative to support B is 10-x.

- Moment equilibrium equation about B (MB=0):

Taking counterclockwise as positive, the reaction Ay creates a clockwise (negative) moment and the unit load 1 creates a counterclockwise (positive) moment about B:

-Ay⋅10+1⋅10-x=0

- Isolating Ay:

10⋅Ay=10-x

Ay=10-x10=1-110x

Key Point Verification

- x=0 m: Ay=1-010=1

- x=2.5 m: Ay=1-2.510=0.75

- x=5 m: Ay=1-510=0.5

- x=7.5 m: Ay=1-7.510=0.25

- x=10 m: Ay=1-1010=0

Fig 6

Example 02

Construct the influence line for the vertical reaction at B of the beam in Fig. 7.

Fig 7

Soltution

Tabulate Values.

Statics verification for the reaction By at each load position x using moment equilibrium about support A (MA=0, counterclockwise positive):

Step-by-Step Statics Calculations

- x=0 m (Support A):

MA=0⟹By5-10=0⟹By=0

- x=2.5 m (Mid-span AB):

MA=0⟹By5-12.5=0⟹By=2.55=0.5

- x=5.0 m (Support B):

MA=0⟹By5-15=0⟹By=55=1.0

- x=7.5 m (Mid-overhang):

MA=0⟹By5-17.5=0⟹By=7.55=1.5

- x=10.0 m (Free End):

MA=0⟹By5-110=0⟹By=105=2.0

Tabulated Values for By

| Position x (m) | Moment Equilibrium Equation | Reaction By​ |
| --- | --- | --- |
| 0.0 | By5-10=0 | 0.0 |
| 2.5 | By5-12.5=0 | 0.5 |
| 5.0 | By5-15=0 | 1.0 |
| 7.5 | By5-17.5=0 | 1.5 |
| 10.0 | By5-110=0 | 2.0 |

Influence Line Equation for By

Influence-Line Equation. We must determine the reaction By when the unit load is placed a distance x from A, Fig. 7. Applying moment equilibrium about support A (MA=0, taking counterclockwise moments as positive) yields the continuous equation for By across the entire span Fig 8 : 0≤x≤10 m:

MA=0⟹By⋅5-1⋅x=0

Byx=x5=0.2x for 0≤x≤10 m

Fig 8

Example 3

Construct the influence line for the shear at point C of the beam in Fig. 9.

Fig 9

Solution

Tabulate Values. At each selected position x of the unit load, the method of sections is used to calculate the value of  Vc. Note in particular that the unit load must be placed just to the left (x=2.5-) and just to the right (x=2.5+) of point C since the shear is discontinuous at C, Figs. 10 and 11. A plot of the values in Table 3 yields the influence line for the shear at C, Fig. 13. Here the diagram gives the shear at C as the unit load moves from one position to the next along the beam.

Internal shear force VC calculation using static equilibrium equations (MA=0 and Fy=0) for each unit load position along the beam (L=10 ft=3.048 m) with section cut at xC=2.5 ft=0.762 m:

1. Load at x=0 ft (0 m)

- Support Reactions:

MA=0⟹By10-10=0⟹By=0

Fy=0⟹Ay+By-1=0⟹Ay=1

- Equilibrium of Left Segment AC:

Fy=0⟹Ay-1-VC=0⟹1-1-VC=0⟹VC=0

2. Load at x=2.5- ft (0.762- m) — Just to the left of point C

- Support Reactions:

MA=0⟹By10-12.5=0⟹By=0.25

Fy=0⟹Ay=0.75

- Equilibrium of Right Segment CB:

Fy=0⟹VC+By=0⟹VC+0.25=0⟹VC=-0.25

3. Load at x=2.5+ ft (0.762+ m) — Just to the right of point C

- Support Reactions:

MA=0⟹By10-12.5=0⟹By=0.25

Fy=0⟹Ay=0.75

- Equilibrium of Left Segment AC:

Fy=0⟹Ay-VC=0⟹0.75-VC=0⟹VC=+0.75

4. Load at x=5 ft (1.524 m)

- Support Reactions:

MA=0⟹By10-15=0⟹By=0.50

Fy=0⟹Ay=0.50

- Equilibrium of Left Segment AC:

Fy=0⟹Ay-VC=0⟹0.50-VC=0⟹VC=+0.50

5. Load at x=7.5 ft (2.286 m)

- Support Reactions:

MA=0⟹By10-17.5=0⟹By=0.75

Fy=0⟹Ay=0.25

- Equilibrium of Left Segment AC:

Fy=0⟹Ay-VC=0⟹0.25-VC=0⟹VC=+0.25

6. Load at x=10 ft (3.048 m)

- Support Reactions:

MA=0⟹By10-110=0⟹By=1.00

Fy=0⟹Ay=0

- Equilibrium of Left Segment AC:

Fy=0⟹Ay-VC=0⟹0-VC=0⟹VC=0

Fig 11

Fig 10

Table 3

Influence-Line Equations.

Here two equations have to be determined, since there are two segments for the influence line due to the discontinuity of shear at C. First we determine Ay, Fig 12. Using the result we then obtain the internal shear at C for each region, Fig. 13. The equations are plotted in Fig.14.

Fig 12

Fig 15

Fig 13

Example 04

Construct the influence line for the shear at point C of the beam in Fig. 15

Fig 14

Tabluate values

Calculs Point par Point

- x=0 m (Load at support A / Charge en A) :

  - MB=0⟹RA⋅8-1⋅8=0⟹RA=1,00

  - Fy=0⟹RB=1-1,00=0,00

  - Shear force / Effort tranchant : VC=RA-1=1,00-1=0,00

- x=2 m (Load between A and C / Charge entre A et C) :

  - MB=0⟹RA⋅8-1⋅6=0⟹RA=0,75

  - Fy=0⟹RB=1-0,75=0,25

  - VC=RA-1=0,75-1=-0,25

- x=4- m (Load just left of C / Charge à gauche de C) :

  - RA=0,50, RB=0,50, VC=RA-1=-0,50

- x=4+ m (Load just right of C / Charge à droite de C) :

  - RA=0,50, RB=0,50, VC=RA=+0,50

- x=6 m (Load between C and B / Charge entre C et B) :

  - RA=0,25, RB=0,75, VC=RA=+0,25

- x=8 m (Load at support B / Charge en B) :

  - RA=0,00, RB=1,00, VC=0,00

- x=10 m (Load on overhang / Charge sur le porte-à-faux) :

  - MA=0⟹RB⋅8-1⋅10=0⟹RB=1,25

  - RA=1-1,25=-0,25⟹VC=-0,25

- x=12 m (Load at free end / Charge à l'extrémité libre) :

  - RB=1,50, RA=-0,50⟹VC=-0,50

Tableau Récapitulatif (Summary Table)

| Position x (m) | Reaction RA​ | Reaction RB​ | Shear Force VC​ |
| --- | --- | --- | --- |
| 0 | 1.00 | 0.00 | 0.00 |
| 2 | 0.75 | 0.25 | -0.25 |
| 4- | 0.50 | 0.50 | -0.50 |
| 4+ | 0.50 | 0.50 | +0.50 |
| 6 | 0.25 | 0.75 | +0.25 |
| 8 | 0.00 | 1.00 | 0.00 |
| 10 | -0.25 | 1.25 | -0.25 |
| 12 | -0.50 | 1.50 | -0.50 |

Influence-Line Equations.

Support Reactions Equations (Équations des réactions d'appui)

- Moment equilibrium about B (MB=0) [Équilibre des moments en B] :

RAx⋅8-1⋅8-x=0⟹RAx=1-x8

- Vertical force equilibrium (Fy=0) [Équilibre des forces verticales] :

RAx+RBx-1=0⟹RBx=x8

Influence Line Equations for Shear Force VC (Équations de l'effort tranchant en C)

- Segment 1 (0≤x<4 m) — Unit load to the left of C (Charge à gauche de C) :

VCx=RAx-1=1-x8-1=-x8

- Segment 2 (4<x≤12 m) — Unit load to the right of C (Charge à droite de C) :

VCx=RAx=1-x8

Summary Table (Tableau des valeurs)

| Position x (m) | Segment / Condition | Equation for VC​ | Calculation | Shear Force VC​ |
| --- | --- | --- | --- | --- |
| 0 | 0≤x<4 | VC=-x8 | -08 | 0.00 |
| 2 | 0≤x<4 | VC=-x8 | -28 | -0.25 |
| 4- | Just left of C | VC=-x8 | -48 | -0.50 |
| 4+ | Just right of C | VC=1-x8 | 1-48 | +0.50 |
| 6 | 4<x≤12 | VC=1-x8 | 1-68 | +0.25 |
| 8 | 4<x≤12 | VC=1-x8 | 1-88 | 0.00 |
| 10 | 4<x≤12 | VC=1-x8 | 1-108 | -0.25 |
| 12 | 4<x≤12 | VC=1-x8 | 1-128 | -0.50 |

Fig 16

Example 05

Construct the influence line for the moment at point C of the beam in Fig. 17.

Fig 17

Solution

Tabulate values

At each selected position of the unit load, the value of MC is calculated using the method of sections. For example, see Fig. 18 for x=2.5 ft. A plot of the values in Table 04 yields the influence line for the moment at C figure 19. This diagram gives the moment at C as the unit load moves from one position to the next along the beam.

Fig 18

Direct Point-by-Point Calculations (Calculs directs point par point)

- x=0 ft (Load at support A / Charge en A) :

  - MB=0⟹RA⋅10-1⋅10=0⟹RA=1.00

  - Fy=0⟹RB=1-1.00=0.00

  - Bending moment / Moment fléchissant : MC=RB⋅5=0.00⋅5=0.00 ft

- x=2.5 ft (Load between A and C / Charge entre A et C) :

  - MB=0⟹RA⋅10-1⋅7.5=0⟹RA=0.75

  - Fy=0⟹RB=1-0.75=0.25

  - Bending moment / Moment fléchissant : MC=RB⋅5=0.25⋅5=1.25 ft

- x=5 ft (Load at point C / Charge en C) :

  - MB=0⟹RA⋅10-1⋅5=0⟹RA=0.50

  - Fy=0⟹RB=1-0.50=0.50

  - Bending moment / Moment fléchissant : MC=RA⋅5=0.50⋅5=2.50 ft

- x=7.5 ft (Load between C and B / Charge entre C et B) :

  - MB=0⟹RA⋅10-1⋅2.5=0⟹RA=0.25

  - Fy=0⟹RB=1-0.25=0.75

  - Bending moment / Moment fléchissant : MC=RA⋅5=0.25⋅5=1.25 ft

- x=10 ft (Load at support B / Charge en B) :

  - MB=0⟹RA⋅10-1⋅0=0⟹RA=0.00

  - Fy=0⟹RB=1-0.00=1.00

  - Bending moment / Moment fléchissant : MC=RA⋅5=0.00⋅5=0.00 ft

Summary Table 04 (Tableau Récapitulatif)

| Position x (ft) | Reaction RA​ | Reaction RB​ | Bending Moment MC​ (ft) |
| --- | --- | --- | --- |
| 0 | 1.00 | 0.00 | 0.00 |
| 2.5 | 0.75 | 0.25 | 1.25 |
| 5 | 0.50 | 0.50 | 2.50 |
| 7.5 | 0.25 | 0.75 | 1.25 |
| 10 | 0.00 | 1.00 | 0.00 |

Fig 19

Influence-Line Equations.

The two line segments for the influence line can be determined using Mc = 0 along with the method of sections shown in Fig. 19.

Support Reactions Equations

- Moment equilibrium about support B (MB=0) :

RAx⋅10-1⋅10-x=0⟹RAx=1-x10

- Vertical force equilibrium (Fy=0) :

RAx+RBx-1=0⟹RBx=x10

Bending Moment Equations for MC (Équations du moment fléchissant en C)

- Segment 1 (0≤x≤5 ft) — Unit load to the left of C (figure 20.a):

MCx=RBx⋅5=x10⋅5=0.5x

- Segment 2 (5≤x≤10 ft) — Unit load to the right of C (figure  20.b) :

MCx=RAx⋅5=1-x10⋅5=5-0.5x

Fig 20.a

Fig 20.b

Summary Table (Tableau récapitulatif par équations)

| Position x (ft) | Segment / Condition | Equation for MC​ | Calculation | Reaction RA​ | Reaction RB​ | Bending Moment MC​ (ft) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0≤x≤5 | MC=0.5x | 0.5⋅0 | 1.00 | 0.00 | 0.00 |
| 2.5 | 0≤x≤5 | MC=0.5x | 0.5⋅2.5 | 0.75 | 0.25 | 1.25 |
| 5 | Point C | MC=0.5x | 0.5⋅5 | 0.50 | 0.50 | 2.50 |
| 7.5 | 5≤x≤10 | MC=5-0.5x | 5-0.5⋅7.5 | 0.25 | 0.75 | 1.25 |
| 10 | 5≤x≤10 | MC=5-0.5x | 5-0.5⋅10 | 0.00 | 1.00 | 0.00 |

Example 06

Construct the influence line for the moment at point C of the beam in Fig. 21

Fig 21

SOLUTION

Tabulate Values. Using statics and the method of sections, verify that the values of the moment MC at point C in Fig. 21 correspond to each position x of the unit load. A plot of the values in Fig. 22.b yields the influence line in Fig. 22.c.

Fig 22

Influence-Line Equations.

Support Reactions Equations (Équations des réactions d'appui)

- Moment equilibrium about B (MB=0) [Équilibre des moments en B] :

RAx⋅8-1⋅8-x=0⟹RAx=1-x8

- Vertical force equilibrium (Fy=0) [Équilibre des forces verticales] :

RAx+RBx-1=0⟹RBx=x8

Influence Line Equations for Bending Moment MC (Équations du moment fléchissant)

- Segment 1 (0≤x≤4 m) — Unit load to the left of C (Charge à gauche de C) :

MCx=RBx⋅4=x8⋅4=0.5x

- Segment 2 (4≤x≤12 m) — Unit load to the right of C (Charge à droite de C) :

MCx=RAx⋅4=1-x8⋅4=4-0.5x

Summary Table (Tableau Récapitulatif)

| Position x (m) | Segment / Condition | Equation for MC​ | Reaction RA​ | Reaction RB​ | Bending Moment MC​ (m) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0≤x≤4 | MC=0.5⋅0 | 1.00 | 0.00 | 0.00 |
| 2 | 0≤x≤4 | MC=0.5⋅2 | 0.75 | 0.25 | 1.00 |
| 4 | Point C | MC=0.5⋅4 | 0.50 | 0.50 | 2.00 |
| 6 | 4≤x≤12 | MC=4-0.5⋅6 | 0.25 | 0.75 | 1.00 |
| 8 | Support B | MC=4-0.5⋅8 | 0.00 | 1.00 | 0.00 |
| 10 | Overhang (Console) | MC=4-0.5⋅10 | -0.25 | 1.25 | -1.00 |
| 12 | Free End (Extrémité libre) | MC=4-0.5⋅12 | -0.50 | 1.50 | -2.00 |

I.3 MÜLLER-BRESLAU PRINCIPLE : QUALITATIVE KINEMATIC APPROACH

In 1886, Heinrich Müller-Breslau developed a technique for quickly determining the shape of an influence line.

The Müller-Breslau principle is based on the principle of virtual work and provides a simple way to construct the qualitative shape of influence lines.

Principle

The qualitative shape of the influence line for a given function, such as:

- Support reaction (R)

- Shear force (T) or (V)

- Bending moment (M)

can be obtained by:

- Releasing the restraint corresponding to the function being studied at the section where the influence line is required.

- Applying a unit action corresponding to this function in its positive direction.

- Allowing the modified structure to deform.

The deflected shape of the modified structure represents the qualitative shape of the influence line.

Easy rule to remember

RELEASE → APPLY → DEFORM → INFLUENCE LINE

Example 1: Influence Line for the Vertical Reaction (Ay)

Consider the beam shown in Fig. 23.a.

Fig 23

We want to determine the qualitative shape of the influence line for the vertical reaction (Ay) at A.

Step 1 – Release the reaction

Replace the pin at A with a roller guide, as shown in Fig. 23.b.

The roller guide still allows the beam to resist the horizontal force, but it cannot resist the vertical reaction (Ay).

Step 2 – Apply the positive action

Apply a positive upward force (Ay) at A.

Step 3 – Observe the deformation

The beam deflects into the dashed position shown in Fig. 23.c.

Conclusion

The dashed deflected shape in Fig. 23.c represents the qualitative shape of the influence line for (Ay).

Remember:
Release (Ay) → Apply (+Ay) → Observe the deflected shape → Influence line for (Ay).

Example 2: Influence Line for the Shear Force at C

Let us determine the qualitative shape of the influence line for the shear force (VC) at section (C), as shown in Fig. 24.a.

Step 1 – Release the shear

The section at (C) is replaced by a roller guide, as shown in Fig. 24.b.

This support can resist moment and axial force, but it cannot resist shear force.

Step 2 – Apply the positive shear

Apply a positive shear force (VC) at point (C).

Step 3 – Observe the deformation

The beam deforms into the dotted shape shown in Fig. 24.b.

Step 4 – Obtain the influence line

This deformed shape gives the qualitative shape of the influence line for (VC), as shown in Fig. 24.c.

Conclusion

Release the shear → Apply (+VC) → Observe the deformation → Obtain the influence line for (VC).

Fig 24

Example 3: Influence Line for the Bending Moment at C

Let us determine the qualitative shape of the influence line for the bending moment MCat section C, as shown in Fig. 25.a.

Step 1 – Release the moment

Introduce an internal hinge (articulation) at C, as shown in Fig. 25.b.

The internal hinge can resist axial and shear forces, but it cannot resist a bending moment.

Step 2 – Apply the positive moment

Apply a positive bending moment MCat C.

Step 3 – Observe the deformation

The beam deforms into the dotted position shown in Fig. 25.b.

Step 4 – Obtain the influence line

The resulting deformed shape represents the qualitative shape of the influence line for MC, as shown in Fig. 25.c.

Conclusion

Release the moment → Apply +MC→ Observe the deformation → Obtain the influence line for MC.

Fig 25

Exmaple 4

For each beam in Fig. 26.a through 26.c, sketch the influence line for the vertical reaction at A

SOLUTION

The support is replaced by a roller guide at A since it will resist Ax, but not Ay. The force Ay is then applied.

Fig 26

Again, a roller guide is placed at A and the force Ay is applied

Fig 26

A double-roller guide must be used at A in this case, since this type of support will resist both a moment MA at the fixed support and axial load Ax, but will not resist Ay.

Fig 26

Example 05

For each beam in Figs. 27.a through 27.c, sketch the influence line for the shear at B.

SOLUTION

The roller guide is introduced at B and the positive shear VB is applied. Notice that the right segment of the beam will not deflect since the roller at A actually constrains the beam from moving vertically, either up or down

Fig 27

Placing the roller guide at B and applying the positive shear at B yields the deflected shape and corresponding influence line.

Fig 27

Again, the roller guide is placed at B, the positive shear is applied, and the deflected shape and corresponding influence line are shown. Note that the left segment of the beam does not deflect, due to the fixed support.

Fig 27

Example 06

For each beam in Figs. 28.a through Fig. 28.c, sketch the influence line for the moment at B.

SOLUTION

A hinge is introduced at B and positive moments MB are applied to the beam. The deflected shape and corresponding influence line are shown.

Fig 28

Placing a hinge at B and applying positive moments MB to the beam yields the deflected shape and influence line.

With the hinge and positive moment at B the deflected shape and influence line are shown. The left segment of the beam is constrained from moving due to the fixed wall at A

I.4. Influence Lines for Floor Girders

RA

RD

FB

FC

In a floor system, loads are transferred from the slab to the floor beams, then to the girder, and finally to the columns. The girder is simply supported by the columns. As shown in Fig. 29.a, the load is transferred to the girder at specific points A, B, C, and D, called panel points. The part between two panel points, such as B–C in Fig. 29.b, is called a panel. These points are important for drawing the influence lines of the girder.

The influence line for a specified point on the girder can be determined using the same statics procedure, place the unit load at various points x on the floor slab and always compute the function (shear or moment) at the specified point P in the girder, Fig. 29.b. Plotting these values versus x yields the influence line for the function at P.

For example, when the unit load is placed on the floor slab, as shown in Fig. 29.b:

- Determine the slab reactions FB and FC.

- Use these reactions to calculate the girder support reactions FA and FD.

- Cut the girder at point Pand use the method of sections to determine the internal shear VP and internal moment MP .

VP =RA -FB

MP ​= RA x−FB​(x−s)

where:

- VP= internal shear at point P

- MP= internal moment at point P

- RA= girder reaction at A

- FB= reaction from the floor slab at B

- x= distance from Ato P

- s= distance from Ato B

Important : VPis constant within the panel BC, whereas MPdepends on the exact position of P.

Step-by-Step Analysis

1. Slab Reactions (FB and FC)

For a unit load acting on slab panel BC at distance x from A:

- Span of the panel segment = s

- Position of unit load relative to point B = x-s

Applying static equilibrium to the slab span BC:

- MC=0⟹FB⋅s=1⋅2s-x

FB=2s-xs=2-xs

- MB=0⟹FC⋅s=1⋅x-s

FC=x-ss=xs-1

Girder Support Reactions (RA and RD)

The total length of the girder is L=3s. The loads transmitted from the slab through the floor beams onto the girder are FB at point B (x=s) and FC at point C (x=2s).

Applying equilibrium to the entire girder:

- Taking moments about D (x=3s):

MD=0⟹RA3s=FB2s+FCs

RA3s=2-xs2s+xs-1s=3s-x

RA=3s-x3s=1-x3s

- Taking moments about A (x=0):

MA=0⟹RD3s=FBs+FC2s

RD3s=2-xss+xs-12s=x

RD=x3s

- Internal Forces at Point P (VP and MP)

Consider a section cut at point P located at distance d from support A (where s<d<2s). Isolate the left segment of the girder (A to P) Fig29.c and Fig 29.d:

- Internal Shear (VP):

Fy=0⟹RA-FB-VP=0

VP=RA-FB=1-x3s-2-xs=-1+2x3s

(Note: VP depends only on the unit load position x, remaining uniform for any point P within panel BC.)

- Internal Moment (MP):

MP=0⟹MP=RA⋅d-FB⋅d-s

MP=1-x3sd-2-xsd-s

MP=2s-d-x+2xd3s

Here is the calculation summary for the three unit load positions (measuring distance x from support A):

Case 1: Unit Load at Point B (x=s)

- Slab Reactions:

  - FB=2-ss=1

  - FC=ss-1=0

- Girder Support Reactions:

  - RA=1-s3s=23

  - RD=s3s=13

- Internal Forces at Point P:

  - Shear force: VP=-1+2s3s=-13

  - Bending moment: MP=23d-1d-s=s-13d

Intermediate Case: Unit Load Between B and C (Midpoint of Panel, x=1.5s=32s)

The midpoint of panel BC is located at distance x=s+0.5s=1.5s from support A.

- Slab Reactions:

  - FB=2-1.5ss=12

  - FC=1.5ss-1=12

- Girder Support Reactions:

  - RA=1-1.5s3s=12

  - RD=1.5s3s=12

- Internal Forces at Point P:

  - Shear force: VP=RA-FB=12-12=0

  - Bending moment: MP=12d-12d-s=12s

Case 2: Unit Load at Point C (x=2s)

- Slab Reactions:

  - FB=2-2ss=0

  - FC=2ss-1=1

- Girder Support Reactions:

  - RA=1-2s3s=13

  - RD=2s3s=23

- Internal Forces at Point P:

  - Shear force: VP=-1+22s3s=13

  - Bending moment: MP=13d-0d-s=13d

What this means for the Influence Line (IL)Because the transmitted panel loads FB and Fc  vary linearly (as straight lines) between B and C, any structural effect in the main girder (support reactions, shear force VP, or bending moment MP) must also vary linearly as a straight line across panel BC

💡The Golden Rule in Practice

- Thanks to this fundamental property, you never need to set up equations for load positions in the middle of a panel!To draw a complete Influence Line for an indirectly loaded floor system:Calculate the effect only when the unit load is placed at each panel point (A, B, C, D, etc.).Plot these specific values on your diagram.Connect adjacent points using straight line segments.

- As long as the load remains in panel CD, panel BC receives no direct transferred load. The internal shear passing through this entire panel remains completely constant from B to C, equal to the support reaction RA.

Example 01

Draw the influence line for the shear in panel CD of the floor girder in Fig. 30.a.

Derivation of Support Reactions RG and RF

The main girder rests on a roller support at G (x=10 ft) and a pin support at F (x=40 ft). The distance between supports is LGF=40-10=30 ft.

To derive the support reaction formulas for any unit load position x measured from point A:

- Sum of moments about support F (x=40 ft):

MF=0⟹RG⋅40-10-1⋅40-x=0

30RG=40-x⟹RG=40-x30

- Sum of moments about support G (x=10 ft):

MG=0⟹RF⋅40-10-1⋅x-10=0

30RF=x-10⟹RF=x-1030

- Vertical Equilibrium Check:

RG+RF=40-x30+x-1030=3030=1 (Satisfied)

2. General Expression for Panel Shear VCD

To find the internal shear force VCD, make a section cut anywhere inside panel CD (20 ft<xcut<30 ft) and sum the vertical forces acting to the left of the cut:

Fy=0⟹RG-FA-FB-FC-VCD=0

VCD=RG-FA+FB+FC

3. Detailed Step-by-Step Calculations

Case 1: Load at Node A (x=0 ft)

- Reactions:

RG=40-030=4030=43

RF=0-1030=-1030=-13

- Transmitted Floor Loads: FA=1, FB=0, FC=0

- Panel Shear:

VCD=RG-FA=43-1=+13

Case 2: Load at Node B (x=10 ft)

- Reactions:

RG=40-1030=3030=1

RF=10-1030=0

- Transmitted Floor Loads: FA=0, FB=1, FC=0

- Panel Shear:

VCD=RG-FB=1-1=0

Case 3: Load at Node C (x=20 ft)

- Reactions:

RG=40-2030=2030=23

RF=20-1030=1030=13

- Transmitted Floor Loads: FA=0, FB=0, FC=1

- Panel Shear:

VCD=RG-FC=23-1=-13

Case 4: Load at Midpoint of Panel CD (x=25 ft)

- Reactions:

RG=40-2530=1530=12

RF=25-1030=1530=12

- Transmitted Floor Loads (Distributed across Panel CD):

FC=30-2510=510=12

FD=25-2010=510=12

FA=0, FB=0

- Panel Shear:

VCD=RG-FC=12-12=0

Case 5: Load at Node D (x=30 ft)

- Reactions:

RG=40-3030=1030=13

RF=30-1030=2030=23

- Transmitted Floor Loads:

FA=0, FB=0, FC=0, FD=1

- Panel Shear:

(Since FD acts to the right of the section cut, it does not enter the left-hand section equilibrium)

VCD=RG-0=+13

Case 6: Load at Node E (x=40 ft)

- Reactions:

RG=40-4030=0

RF=40-1030=3030=1

- Transmitted Floor Loads:

FA=0, FB=0, FC=0, FE=1

- Panel Shear:

VCD=RG-0=0

4. Summary Table

| Load Position x | Panel Point | RG​ | RF | FFC | Panel Shear VCD​ |
| --- | --- | --- | --- | --- | --- |
| 0 ft | A | +4/3 | -1/3 | 0 | +1/3 |
| 10 ft | B (G) | 1 | 0 | 0 | 0 |
| 20 ft | C | +2/3 | +1/3 | 1 | -1/3 |
| 25 ft | Midpoint CD | +1/2 | +1/2 | 1/2 | 0 |
| 30 ft | D | +1/3 | +2/3 | 0 | +1/3 |
| 40 ft | E (F) | 0 | 1 | 0 | 0 |

Influence Line. When the tabular values are plotted and the points connected with straight line segments, the resulting influence line for VCD is as shown in Fig. 30.e.

Example 2

Draw the influence line for the moment at point F for the floor girder in Fig. 31.

Solution

Static Model and System Geometry

From the structural diagram:

- Origin (x=0): Set at the leftmost end at node A.

- Panel Points (Floor Beams / Load Transfer Nodes):

  - Node A: xA=0 m (rests directly on the abutment/wall)

  - Node B: xB=4 m

  - Node C: xC=8 m

  - Node D: xD=12 m

  - Node E: xE=16 m

- Main Girder Supports:

  - Pin support H: xH=2 m

  - Roller support G: xG=16 m

  - Main girder span: L=xG-xH=16-2=14 m

- Section of Interest F: xF=10 m (located 8 m from H and 6 m from G).

2. General Support Reaction Equations

For a concentrated force P acting on the main girder at position xgirder (measured from A):

- Sum of moments about support H (x=2 m):

MH=0⟹RG⋅14-P⋅xgirder-2=0⟹RG=P⋅xgirder-214

- Sum of moments about support G (x=16 m):

MG=0⟹RH⋅14-P⋅16-xgirder=0⟹RH=P⋅16-xgirder14

3. Principle of Indirect Loading

In an indirectly loaded floor system, the unit moving load (1 kN) travels across the slab, which transfers concentrated loads exclusively to the panel points (A,B,C,D,E).

Between adjacent panel points, any internal structural effect varies strictly linearly (straight line segments). Therefore, we only need to calculate MF by placing the 1 kN unit load successively at each panel point.

4. Node-by-Node Detailed Calculations

Case 1: Unit Load at Node A (x=0 m)

- The load is applied directly above the abutment wall.

- No force is transmitted to the main girder (FB=FC=FD=FE=0).

- Support reactions: RH=0, RG=0.

- Moment at F:

MF=0 kN⋅m

Case 2: Unit Load at Node B (x=4 m)

- The floor slab transfers the entire load to node B: FB=1 kN at x=4 m.

- Support reaction at G:

RG=1⋅4-214=214=17 kN

- Cut section at point F (xF=10 m) and consider the right segment (F→G):

MF=RG⋅16-10=17⋅6=+67 kN⋅m≈+0.857 kN⋅m

Case 3: Unit Load at Node C (x=8 m)

- The floor slab transfers the entire load to node C: FC=1 kN at x=8 m.

- Support reaction at G:

RG=1⋅8-214=614=37 kN

- Cut section at point F (xF=10 m) and consider the right segment (F→G):

MF=RG⋅16-10=37⋅6=+187 kN⋅m≈+2.571 kN⋅m

Case 4: Unit Load at Node D (x=12 m)

- The floor slab transfers the entire load to node D: FD=1 kN at x=12 m.

- Support reaction at H:

RH=1⋅16-1214=414=27 kN

- Cut section at point F (xF=10 m) and consider the left segment (H→F):

MF=RH⋅10-2=27⋅8=+167 kN⋅m≈+2.286 kN⋅m

Case 5: Unit Load at Node E (x=16 m)

- The load acts directly over support G (RG=1 kN, RH=0).

- Cut section at point F and consider the left segment:

MF=RH⋅10-2=0⋅8=0 kN⋅m

5. Verification at Intermediate Position (x=10 m)

When the unit load is directly over point F (x=10 m, midpoint of panel CD):

- Load distribution to nodes C and D:

FC=12-104=12 kN, FD=10-84=12 kN

- Support reaction at G:

RG=12⋅8-214+12⋅12-214=314+514=814=47 kN

- Moment at F (right segment F→G):

MF=RG⋅16-10-FD⋅12-10=47⋅6-12⋅2=247-1=+177 kN⋅m≈+2.429 kN⋅m

(Note: This matches the arithmetic mean between MFC=187 and MFD=167, confirming linear continuity across the panel).

6. Summary Table

| Load Position (x) | Panel Point | Reaction RH​ | Reaction RG​ | Moment MF​ (kN⋅m) |
| --- | --- | --- | --- | --- |
| x=0 m | A | 0 | 0 | 0 |
| x=4 m | B | +6/7 | +1/7 | +67≈+0.857 |
| x=8 m | C | +4/7 | +3/7 | +187≈+2.571 (Maximum) |
| x=10 m | Midpoint CD (F) | +3/7 | +4/7 | +177≈+2.429 |
| x=12 m | D | +2/7 | +5/7 | +167≈+2.286 |
| x=16 m | E | 0 | 1 | 0 |

7. Plotting Instructions

- Horizontal Baseline: Draw a reference axis from x=0 to x=16 m marking nodes A,B,C,D,E.

- Plot Key Ordinates:

  - At A (x=0): 0

  - At B (x=4 m): +6/7

  - At C (x=8 m): +18/7

  - At D (x=12 m): +16/7

  - At E (x=16 m): 0

- Connect Segments: Join adjacent points sequentially (A→B→C→D→E) using straight line segments:

  - A→B: Increasing line from 0 to +6/7.

  - B→C: Increasing line from +6/7 to the maximum peak at +18/7.

  - C→D: Continuous decreasing line from +18/7 to +16/7 (passing through +17/7 at x=10 m).

  - D→E: Decreasing line from +16/7 back to 0.

I.5 Influence Lines for Trusses

Trusses are often used as primary load-carrying elements for bridges. Hence, for design it is important to be able to construct the influence lines for each of its members. As shown in                  Fig. 33, the loading on the bridge deck is transmitted to stringers, which in turn transmit the loading to floor beams and then to the joints along the bottom cord of the truss.

Trusses are commonly used to support bridge loads. To draw the influence line of a truss member, a unit load is placed successively at each joint along the bottom chord. The force in the member is then calculated using the method of joints or method of sections. The calculated forces are plotted against the load positions. Tension is taken as positive (+) and compression as negative (−). Finally, the points are connected with straight lines to obtain the influence line.

Exercice 01

Draw the influence line diagrams for the axial forces in members IJ, CD, and ID of the bridge truss shown in Fig. 6–24a.

To plot the influence lines for the axial forces in members IJ, CD, and ID, we use Method of Sections (Ritter's method).

1. Geometry and Support Reactions

- Dimensions:

  - 6 panels of 5 m each ⟹ Total length =30 m.

  - Truss height h=4 m.

  - Node coordinates along the bottom chord:

    - A=0 m (Support A)

    - B=5 m

    - C=10 m

    - D=15 m

    - E=20 m (Support E)

    - F=25 m

    - G=30 m (Cantilever end from E to G)

- Support Reactions for a moving downward unit load P=1 at position x∈0,30 m:

ME=0⟹RAx=20-x20

MA=0⟹REx=x20

2. Influence Line for Axial Force NIJ (Top Chord)

Pass a section cutting through members IJ, ID, and CD. Taking moments about node D (xD=15 m,yD=0):

- Load to the right of D (15 m≤x≤30 m):

Isolating the left section:

MD=0⟹RA⋅15+NIJ⋅4=0⟹NIJx=-154RAx=3x-2016

- Load to the left of D (0≤x≤15 m):

Isolating the right section:

MD=0⟹RE⋅5+NIJ⋅4=0⟹NIJx=-54REx=-x16

Key Values for NIJ:

| Position x (m) | 0 (A) | 5 (B) | 10 (C) | 15 (D) | 20 (E) | 25 (F) | 30 (G) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NIJ | 0 | -0.3125 | -0.625 | -0.9375 | 0 | +0.9375 | +1.875 |

- Diagram Shape: A negative triangular shape from A0 to E20 peaking at D15 with -0.9375, followed by a straight upward line reaching +1.875 at G30.

3. Influence Line for Axial Force NCD (Bottom Chord)

Taking moments about node I (xI=10 m,yI=4 m):

- Load to the left of C (0≤x≤10 m):

Isolating the right section:

MI=0⟹RE⋅10-NCD⋅4=0⟹NCDx=104REx=x8

- Load to the right of C (10 m≤x≤30 m):

Isolating the left section:

MI=0⟹RA⋅10-NCD⋅4=0⟹NCDx=104RAx=20-x8

Key Values for NCD:

| Position x (m) | 0 (A) | 5 (B) | 10 (C) | 15 (D) | 20 (E) | 25 (F) | 30 (G) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NCD | 0 | +0.625 | +1.25 | +0.625 | 0 | -0.625 | -1.25 |

- Diagram Shape: A positive triangle from A0 to E20 peaking at C10 with +1.25, then sloping downward to -1.25 at G30.

4. Influence Line for Axial Force NID (Diagonal)

Diagonal ID connects node I10,4 to D15,0.

- Inclination: sinα=hLID=452+42=441≈0.6247.

Applying vertical force equilibrium Fy=0 across the section:

- Load to the left of panel CD (0≤x≤10 m):

Isolating the right section:

Fy=0⟹RE+NIDsinα=0⟹NIDx=-RExsinα=-4180x≈-0.08004 x

- Load to the right of panel CD (15 m≤x≤30 m):

Isolating the left section:

Fy=0⟹RA-NIDsinα=0⟹NIDx=RAxsinα=418020-x≈0.08004 20-x

- Inside panel CD (10 m≤x≤15 m):

Linear transition between x=10 m and x=15 m, crossing zero (NID=0) at x0=13.33 m (i.e., x=403 m).

Key Values for NID:

| Position x (m) | 0 (A) | 10 (C) | 13.33 | 15 (D) | 20 (E) | 30 (G) |
| --- | --- | --- | --- | --- | --- | --- |
| NID (Exact) | 0 | -418 | 0 | +4116 | 0 | -418 |
| NID (Decimal) | 0 | -0.800 | 0 | +0.400 | 0 | -0.800 |

- Diagram Shape:

  - From A0 to C10: linearly decreases from 0 to -0.800.

  - From C10 to D15: linear transition line passing through 0 at x=13.33 m up to +0.400 at D15.

  - From D15 to G30: straight line going through 0 at E20 and down to -0.800 at G30.

Plotting Summary

| Node / Position x | A (0 m) | C (10 m) | D (15 m) | E (20 m) | G (30 m) |
| --- | --- | --- | --- | --- | --- |
| NIJ | 0 | -0.625 | -0.938 | 0 | +1.875 |
| NCD | 0 | +1.250 | +0.625 | 0 | -1.250 |
| NID | 0 | -0.800 | +0.400 | 0 | -0.800 |

Example 02

Draw the influence line for the force in member CG of the bridge truss shown in Fig. 34.

Solution

. Static Analysis of Joint C (Method of Joints)

To determine the force FCG in the vertical member CG, isolate joint C:

- Members meeting at joint C:

  - Two collinear horizontal bottom-chord members: CB and CD.

  - One vertical member: CG.

- External loading:

  - As the unit moving load P=1 moves across the bridge deck, a fraction PC is transferred to joint C via the floor beams.

Summing forces vertically (Fy=0) at joint C:

Fy=0⟹FCG-PC=0⟹FCG=PC

Thus, the axial force in member CG is directly equal to the fraction of the unit load transmitted to joint C.

2. Calculation of Force FCG for Various Load Positions (x)

Consider a unit load P=1 moving along the bottom chord from x=0 m (joint A) to x=24 m (joint E):

- Load between x=0 m and x=6 m (Panel AB):

  - The load is carried entirely by joints A and B.

  - No load is transmitted to joint C (PC=0).

  - FCG=0

- Load between x=6 m and x=12 m (Panel BC):

  - By the lever rule, the fraction transmitted to joint C increases linearly:

PC=x-612-6=x-66

  - At x=6 m (Joint B): FCG=0

  - At x=12 m (Joint C): FCG=+1 (Tension)

- Load between x=12 m and x=18 m (Panel CD):

  - The fraction transmitted to joint C decreases linearly:

PC=18-x18-12=18-x6

  - At x=12 m (Joint C): FCG=+1 (Tension)

  - At x=18 m (Joint D): FCG=0

- Load between x=18 m and x=24 m (Panel DE):

  - The load is carried entirely by joints D and E.

  - No load is transmitted to joint C (PC=0).

  - FCG=0

3. Summary Table of Ordinates

| Load Position x (m) | Node | Force FCG​ | Stress State |
| --- | --- | --- | --- |
| 0 | A | 0 | Zero force |
| 6 | B | 0 | Zero force |
| 12 | C | +1 | Maximum Tension |
| 18 | D | 0 | Zero force |
| 24 | E | 0 | Zero force |

4. Instructions for Plotting the Influence Line

- Horizontal Baseline: Draw a horizontal axis representing the total span from x=0 m to x=24 m, marking nodes A,B,C,D,E.

- Plot Segments:

  - From x=0 m to x=6 m (A→B): Draw a horizontal line lying on the axis (FCG=0).

  - From x=6 m to x=12 m (B→C): Draw a line sloping upward from 0 to a maximum value of +1 above the axis.

  - From x=12 m to x=18 m (C→D): Draw a line sloping downward from +1 to 0.

  - From x=18 m to x=24 m (D→E): Draw a horizontal line lying on the axis (FCG=0).

I.6 Influence Lines for frame

For a structural frame, influence lines can be used to determine the support reactions, shear force, and bending moment at a specific point. A unit load (W=1)  is moved along the frame, and the corresponding value of the required function is calculated for each load position. These values are then plotted against the load position to obtain the influence line.

Key Points:

- Move a unit load (W=1) along the frame.

- Calculate the desired function (reaction, shear force, or bending moment) at the specified point.

- Plot the calculated values versus the load position (x).

- Connect the key points with straight lines.

Example 01

Consider the frame shown in Fig. 36. The goal is to draw the influence lines for:

a) The vertical reaction at support A (RA)

b) The shear force at point C (VC)

c) The bending moment at point C (MC)

Solution

Global Static Analysis & Reaction Equations

- Frame geometry and supports :

  - The frame consists of a horizontal beam BD with a total span of L=8 m (4 m+4 m) and two vertical columns of height h=3 m.

  - Support A is a pin support (restraining vertical reaction RA and horizontal reaction HA).

  - Support E is a roller support (restraining vertical reaction RE only, HE=0).

  - A downward unit vertical load W=1 moves along the horizontal member BD, with its position defined by distance x measured from joint B (0≤x≤8 m).

- Static equilibrium equations:

  - Horizontal Equilibrium (Fx=0):

HA=0

Since no horizontal external force acts on the frame, the columns carry no horizontal shear, and the frame behaves like a simply supported beam of 8 m span.

  - Moment Equilibrium about Support E (ME=0):

RA⋅8-1⋅8-x=0⟹RAx=1-x8

  - Vertical Equilibrium (Fy=0):

RA+RE=1⟹REx=x8

2. Influence Line Equations & Calculations

a) Influence Line for RA (Vertical Reaction at Support A)

- Equation:

RAx=1-x8 for 0≤x≤8 m

- Key Coordinates:

  - At load position B (x=0 m): RA=1-08=+1.0

  - At load position C (x=4 m): RA=1-48=+0.5

  - At load position D (x=8 m): RA=1-88=0.0

b) Influence Line for VC (Shear Force at Point C)

- Method of Sections at Point C (xC=4 m):

  - Case 1: Load on segment BC (0≤x<4 m):

VCx=RA-1=1-x8-1=-x8

    - At x=0 m (Joint B): VC=0.0

    - At x→4- m (Just left of C): VC=-48=-0.5

  - Case 2: Load on segment CD (4 m<x≤8 m):

VCx=RA=1-x8

    - At x→4+ m (Just right of C): VC=1-48=+0.5

    - At x=8 m (Joint D): VC=1-88=0.0

- Unit Jump at Point C:

ΔVC=VC4+-VC4-=+0.5--0.5=1.0

c) Influence Line for MC (Bending Moment at Point C)

- Sign Convention: Positive bending moment causes sagging (tension at the bottom fibers).

- Equations by Segment:

  - Case 1: Load on segment BC (0≤x≤4 m):

MCx=RE⋅4=x8⋅4=x2

    - At x=0 m (Joint B): MC=0.0 kN⋅m

    - At x=4 m (Point C): MC=42=+2.0 kN⋅m

  - Case 2: Load on segment CD (4 m≤x≤8 m):

MCx=RA⋅4=1-x8⋅4=4-x2

    - At x=4 m (Point C): MC=4-42=+2.0 kN⋅m

    - At x=8 m (Joint D): MC=4-82=0.0 kN⋅m

3. Summary Table of Coordinates (Influence Line Values)

| Load Position (x) | Joint B (0 m) | Point C− (4− m) | Point C+ (4+ m) | Joint D (8 m) |
| --- | --- | --- | --- | --- |
| RA (Vertical Reaction at A) | +1.0 | +0.5 | +0.5 | 0.0 |
| VC (Shear Force at C) | 0.0 | -0.5 | +0.5 | 0.0 |
| MC (Bending Moment at C) | 0.0 kN⋅m | +2.0 kN⋅m | +2.0 kN⋅m | 0.0 kN⋅m |

Example

Solution

Support Reactions (RB and RC)

To determine the influence lines for the support reactions, a unit vertical load P=1 moves along the top beam AD (0≤xp≤8 m).

- Moment equilibrium about Support B (MB=0):

RC⋅4-1⋅xp-2=0⟹RC=xp-24

- Vertical force equilibrium (Fy=0):

RB+RC-1=0⟹RB=6-xp4

- Horizontal reaction at Support B (HB):

HB=0 for all xp

Since no external horizontal forces act on the frame, the horizontal reaction remains zero.

2. Internal Forces at Section x (xp=1 m)

- Shear Force (Vx):

  - For 0≤xp<1 m (load to the left of section x): Vx=-1

  - For 1 m<xp≤8 m (load to the right of section x): Vx=0

- Bending Moment (Mx):

  - For 0≤xp≤1 m: Mx=xp-1

  - For 1 m≤xp≤8 m: Mx=0

3. Shear Force Just Right of Column Joint (VBy)

This corresponds to the shear force in the horizontal beam segment immediately to the right of the joint above column B (xp=2+) :

- For 0≤xp<2 m: VBy=RC=xp-24

- For 2 m<xp≤8 m: VBy=RC-1=xp-64

4. Internal Forces at Section y (xp=4 m)

- Shear Force (Vy):

  - For 0≤xp<4 m: Vy=RC=xp-24

  - For 4 m<xp≤8 m: Vy=RC-1=xp-64

- Bending Moment (My):

  - For 0≤xp≤4 m: My=-2RC=-xp-22

  - For 4 m≤xp≤8 m: My=xp-62

5. Internal Forces at Section z (Vertical Column JB)

- Axial Force (Az / Normal Force):

The vertical column transmits the reaction RB (compression when negative) :

Az=-RB=xp-64

- Column Shear Force (Vz):

Vz=0 for all xp

| Function / Load Position | Point A (xp​=0 m) | Section x (xp​=1 m) | Joint B (xp​=2 m) | Section y (xp​=4 m) | Support C (xp​=6 m) | Point D (xp​=8 m) |
| --- | --- | --- | --- | --- | --- | --- |
| RC (Reaction at C) | -0.5 | -0.25 | 0.0 | +0.5 | +1.0 | +1.5 |
| RB (Reaction at B) | +1.5 | +1.25 | +1.0 | +0.5 | 0.0 | -0.5 |
| Vx (Shear at x) | -1.0 | -1.0∣0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Mx (Moment at x) | -1.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| VBy (Shear right of B) | -0.5 | -0.25 | 0.0/-1.0 | -0.5 | 0.0 | +0.5 |
| Vy (Shear at y) | -0.5 | -0.25 | 0.0 | +0.5/-0.5 | 0.0 | +0.5 |
| My (Moment at y) | +1.0 | +0.5 | 0.0 | -1.0 | 0.0 | +1.0 |
| Az (Axial Force in Column) | -1.5 | -1.25 | -1.0 | -0.5 | 0.0 | +0.5 |
| Vz (Shear in Column) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

6. Summary Table of Influence Line Ordinates

Example 03

Example: Draw the I.L. for the following functions for the frame shown below. RB RC, RAY, Vc, VH, VJ, MH and MJ.

Solution

I.7 Application of Influence Lines :